ETSI mWT SDN Test Plan V1.0 (2025-12)

Disclaimer

The present document has been produced and approved by the millimetre Wave Transmission (mWT) ETSI ATTM TM_mWT Working Group and represents the views of those members who participated in this Working Group.
It does not necessarily represent the views of the entire ETSI membership.

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - APE 7112B

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° w061004871

Important notice

The present document can be downloaded from the
ETSI Search & Browse Standards application.

The present document may be made available in electronic versions and/or in print. The content of any electronic and/or print versions of the present document shall not be modified without the prior written authorization of ETSI. In case of any existing or perceived difference in contents between such versions and/or in print, the prevailing version of an ETSI deliverable is the one made publicly available in PDF format on ETSI deliver repository.

Users should be aware that the present document may be revised or have its status changed, 
this information is available in the Milestones listing.

If you find errors in the present document, please send your comments to
the relevant service listed under Committee Support Staff.

If you find a security vulnerability in the present document, please report it through our

Coordinated Vulnerability Disclosure (CVD) program.

Notice of disclaimer & limitation of liability

The information provided in the present deliverable is directed solely to professionals who have the appropriate degree of experience to understand and interpret its content in accordance with generally accepted engineering or

other professional standard and applicable regulations.

No recommendation as to products and services or vendors is made or should be implied.

No representation or warranty is made that this deliverable is technically accurate or sufficient or conforms to any law and/or governmental rule and/or regulation and further, no representation or warranty is made of merchantability or fitness for any particular purpose or against infringement of intellectual property rights.

In no event shall ETSI be held liable for loss of profits or any other incidental or consequential damages.

Any software contained in this deliverable is provided "AS IS" with no warranties, express or implied, including but not limited to, the warranties of merchantability, fitness for a particular purpose and non-infringement of intellectual property rights and ETSI shall not be held liable in any event for any damages whatsoever (including, without limitation, damages for loss of profits, business interruption, loss of information, or any other pecuniary loss) arising out of or related to the use of or inability to use the software.

Copyright Notification

No part may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm except as authorized by written permission of ETSI.
The content of the PDF version shall not be modified without the written authorization of ETSI.
The copyright and the foregoing restriction extend to reproduction in all media.

© ETSI 2025.

All rights reserved.

Contents

Intellectual Property Rights	4

Foreword	4

Modal verbs terminology	4

1	Scope	5

2	References	5

3	Definition of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	A standardized and interoperable SBI	8

4.1	Introduction	8

4.2	NETCONF Operations Overview	8

5	Hardware Profiles	10

5.1	Introduction	10

5.2	Split-mount (modular) IDU-ODU connectivity, 1+0 configuration	11

5.2.1	HW_CFG_01, Ethernet cable connectivity, ODU is a separate chassis	11

5.2.2	HW_CFG_02, IF cable connectivity, ODU is a module and child component of the indoor	12

5.3	Compact (non-modular) IDU-ODU connectivity, 1+0 configuration	13

5.3.1	HW_CFG_03, Ethernet cable connectivity, ODU is a separate chassis	13

5.3.2	HW_CFG_04, IF cable connectivity, ODU is a module and child component of the indoor	14

6	Test Configurations	15

6.1	Introduction	15

6.2	Single Mode	15

6.3	Dual Mode	16

6.4	Chain Mode	16

7	Use Cases and Tests	18

7.1	Use Case 1: Network and Service Auto-discovery	18

7.1.1	Functionality description	18

7.1.2	Inventory or Network and services auto-discovery tests	18

7.2	USE CASE 2: Service Provisioning	18

7.2.1	Service Description	18

7.2.2	End to End L2 service provisioning tests in Single Mode	18

7.2.3	End to End L2 service provisioning tests in Dual Mode	19

7.2.4	End to End L2 service provisioning tests in Chain Mode	20

7.3	USE Case 3: Performance Analysis & Prediction	21

7.3.1	Functionality description	21

7.3.2	Performance Analysis & Prediction tests	21

7.4	Use Case 4: Smart Alarm Analysis & Fault Prediction	22

7.4.1	Functionality description	22

7.4.2	Smart Alarm Analysis & Fault Prediction tests	22

8	mWT SDN SBI Hardware Inventory Profile	23

# Intellectual Property Rights

Essential patents

IPRs essential or potentially essential to normative deliverables may have been declared to ETSI. The declarations pertaining to these essential IPRs, if any, are publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI IPR online database.

Pursuant to the ETSI Directives including the ETSI IPR Policy, no investigation regarding the essentiality of IPRs, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

Trademarks

The present document may include trademarks and/or tradenames which are asserted and/or registered by their owners. ETSI claims no ownership of these except for any which are indicated as being the property of ETSI, and conveys no right to use or reproduce any trademark and/or tradename. Mention of those trademarks in the present document does not constitute an endorsement by ETSI of products, services or organizations associated with those trademarks.

DECT™, PLUGTESTS™, UMTS™ and the ETSI logo are trademarks of ETSI registered for the benefit of its Members. 3GPP™, LTE™ and 5G™ logo are trademarks of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners. oneM2M™ logo is a trademark of ETSI registered for the benefit of its Members and of the oneM2M Partners. GSM® and the GSM logo are trademarks registered and owned by the GSM Association.

# Foreword

The present document has been produced by ETSI ATTM TM_mWT millimetre Wave Transmission (mWT).

# Modal verbs terminology

In the present document "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the ETSI Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in ETSI deliverables except when used in direct citation.

# Scope

The present document defines the Test Plan for mWT SDN Interoperability events. The Robot Framework is introduced to help automate the testing.

# References

[1]	ETSI GR mWT 016 (July 2017): "Applications and use cases of Software Defined Networking (SDN) as related to microwave and millimetre wave transmission".
https://www.etsi.org/deliver/etsi_gr/mWT/001_099/016/01.01.01_60/gr_mwt016v010101p.pdf

[2]	ETSI GR mWT 025 (March 2021): " Wireless Backhaul Network and Services Automation: SDN SBI YANG models ".
https://www.etsi.org/deliver/etsi_gr/mWT/001_099/025/01.01.01_60/gr_mWT025v010101p.pdf

[3]	ETSI Draft : SDN SBI Wireless Transport – Profiles for HW Inventory Use Cases
https://portal.etsi.org/webapp/WorkProgram/Report_WorkItem.asp?WKI_ID=74377

[4]	IETF RFC 8348 (March 2018): "A YANG Data Model for Hardware Management".
https://datatracker.ietf.org/doc/html/rfc8348

[5]	IETF RFC 7317 (March 2004): "A YANG Data Model for System Management".
https://datatracker.ietf.org/doc/html/rfc7317

[6]	IETF RFC 8561 (June 2019): "A YANG Data Model for Microwave Radio Link ".
https://datatracker.ietf.org/doc/rfc8561/

[7]	IETF RFC 8343 (March 2018): "A YANG Data Model for Interface Management".
https://datatracker.ietf.org/doc/html/rfc8343

[8]	IEEE 802.3.2-2025: "IEEE Standard for Ethernet YANG Data Model Definitions".
https://ieeexplore.ieee.org/document/11216125

NOTE:	Additional resource: https://yangcatalog.org/yang-search/yang_tree/ieee802-ethernet-interface@2019-06-21.

[9]	IEEE 802.1Qcp-2018: "IEEE Standard for Local and metropolitan area networks -- Bridges and Bridged Networks -- Amendment 30: YANG Data Model".
https://standards.ieee.org/ieee/802.1Qcp/6238/

NOTE:	Additional resources:
https://ieee802.org/1/files/public/YANGs/ieee802-dot1q-bridge.yang 
ieee802.org/1/files/public/YANGs/ieee802-dot1q-pb.yang

[10]	IEEE 802.1AB-2016 : "IEEE Standard for Local and metropolitan area networks - Station and Media Access Control Connectivity Discovery".
https://standards.ieee.org/ieee/802.1AB/6047/

NOTE:	Additional resources:
https://www.yangcatalog.org/api/services/tree/ieee802-dot1ab-lldp@2022-03-15.yang.
ieee802.org/1/files/public/YANGs/ieee802-dot1ab-lldp.yang

[11]	IETF RFC 8632 (September 2019): "A YANG Data Model for Alarm Management".
https://datatracker.ietf.org/doc/html/rfc8632

[12]	IETF RFC 6241 (June 2011): "Network Configuration Protocol (NETCONF)".
https://datatracker.ietf.org/doc/html/rfc6241

[13]	IETF RFC 6020 (October 2010): "YANG - A Data Modeling Language for the Network Configuration Protocol (NETCONF)".
https://datatracker.ietf.org/doc/html/rfc6020

[14]	IETF RFC 7950 (August 2016): "The YANG 1.1 Data Modeling Language".
https://datatracker.ietf.org/doc/html/rfc7950

[15]	IETF RFC 8575 (May 2019): "YANG Data Model for the Precision Time Protocol (PTP)".
https://datatracker.ietf.org/doc/html/rfc8575

[16]	draft-ietf-netmod-intf-ext-yang: "Common Interface Extension YANG Data Models".
https://datatracker.ietf.org/doc/draft-ietf-netmod-intf-ext-yang/

[17]	IETF RFC 5277 (July 2008): “NETCONF Event Notifications”
https://datatracker.ietf.org/doc/html/rfc5277

[18]	Robot Framework
https://robotframework.org/

# Definition of terms, symbols and abbreviations

## Terms

Void.

## Symbols

Void.

## Abbreviations

For the purposes of the present document, the following abbreviations apply:

AR	Availability Ratio

BBE	Background Block Error

BCA	Bands and Carrier Aggregation

BER	Bit Error Ratio

BSS	Business Support System

CB	Customer Bridge

CBS	Committed Burst Size

CIR	Committed Information Rate

CM	Current Modulation

C-VLAN	Customer - Virtual Local Area Network

DHCP	Dynamic Host Configuration Protocol

DM	Data Model

EB	Errored Block

EBS	Excess Burst Size

EIR	Excess Information Rate

EMS	Element Management System

ES	Errored Second

FER	Frame Error Ratio

FLR	Frame Loss Ratio

HW	Hardware

IEEE	Institute of Electrical and Electronics Engineers

IETF	 Internet Engineering Task Force

IM	Information Model

IP	Internet Protocol

LLDP	Link Layer Discovery Protocol

LOS	Line Of Sight

MAC	Media Access Control

MSE	Mean Square Error

MTBF	Mean Time Between Failures

MTU	Maximum Transmission Unit

MW	MicroWave

N.A	Not Available

NE	Network Element

NMS	Network Management System

OI	Outage Intensity

ONF	Open Networking Foundation

OPEX	Operational Expenditure

OSS	Operations Support System

PB	Provider Bridge

PEB	Provider Edge Bridge

PCP	Priority Code Point

PEA	Percent Ethernet service Availability

PEU	Percent Ethernet service Unavailability

QoS	Quality of Service

RF	Radio Frequency

RFC	Request For Comment

RSL	Received Signal Level

SBI	SouthBound Interface

SDN	Software Defined Networking

SES	Severely Errored Second

SNR	Signal to Noise Ratio

S-VLAN	 Service - Virtual Local Area Network

TTL	Time To Live

UML	Unified Modeling Language

UR	Unavailability Ratio

VLAN	Virtual Local Area Network

XPIC	 Cross-Polarization Interference Cancelling

YANG	Yet Another Next Generation

# A standardized and interoperable SBI

## Introduction

The Network elements from all participating vendors expose their functions using the NETCONF protocol. The configuration and parameters follow the data models defined in the YANG language.

These YANG models are standardized in respective standards developing organization, including IETF and IEEE.

Each vendor will provide a pair of network elements, forming a Microwave link. This Microwave link is not subject to standardization, but their management is.

The testing methodology uses the Robot Framework [18] which enable automated tests.

Figure 1: SBI and Robot Framework

The Robot Framework [18] exchanges YANG model payloads with the network elements. It pushes XML-formatted data structured according to the selected YANG data models.

The connection between the Robot Framework executing the Python scripts and each network element is performed using Ethernet connections.

## NETCONF Operations Overview

The NETCONF configuration protocol shall be used.

The base protocol includes 9 operations as defined in clause 7 Protocol Operations of RFC 6241 [12].

- <get>

- <get-config>

- <edit-config>

- <copy-config>

- <delete-config>

- <lock>

- <unlock>

- <close-session>

- <kill-session>

These following three NETCONF operations shall be used:

- The <get> operation is used to retrieve the running configuration, as well as the operational state of the network element.

- The <get-config> operation retrieves the configuration from the specified datastore.

- The <edit-config> is used to modify the configuration from the specified datastore.

In addition, the profile requires the :candidate, the :commit-confirmed and the validate capabilities. These 3 capabilities imply the use of four new operations:

- <commit>

- <discard-changes>

- <cancel-commit>

- <validate>

The NETCONF Event Notifications is an optional capability for NETCONF, specified in RFC 5277 [17]. It enables the network element to send event to the controller, following the publish-subscribe architecture. This capability adds two new operations:

- <create-subsscription>

- <notification>

These optional NETCONF operations are tested in the Use Case 4: Smart Alarm Analysis & Fault Prediction, making them required by the profile.

# Hardware Profiles

## Introduction

The ETSI work item "Profiles for HW Inventory Use Cases" [3] defines specific hardware profiles which follow the structure of IETF RFC 8348 [4].

The profiles applicable for the present document are one model for ETH cabling and another for IF cable connectivity between IDU-ODU:

In the case of Eth connectivity between the IDU-ODU, the MW RF ODU is treated as a different chassis by all vendors.

In the case of IF connectivity between the IDU-ODU, the MW RF ODU is treated as child module of the IF port of the modem (part of IDU chassis).

- Furthermore, a distinction is made between modular and fixed (compact, non-modular) indoor unit (IDU) architectures, reflecting the different inventory modelling approaches required for each case:

In the case of a modular IDU, the IDU is modelled as a chassis comprising multiple containers, each of which may host one or more modules. Each module may expose Ethernet (electrical or optical) ports, IF ports, or may itself include additional containers, reflecting the modular and hierarchical nature of the hardware.

In the case of a fixed (compact, non-modular) IDU, the IDU is also modelled as a chassis, but with a fixed set of physical ports. The RJ45 Ethernet ports are modelled as fixed ports of the chassis, as they are permanently integrated and not field-replaceable. SFP cages are modelled as containers capable of hosting pluggable transceiver components. IF/modem interfaces are modelled as containers representing pluggable/activatable modem functions within the IDU. As a result, unlike the modular IDU mode, this representation does not include container slots, since no removable board-level containers are present.

- In the following clauses, the different profiles are defined, using the colouring convention, as shown in the figure below.

Figure 2 Coloring convention

## Split-mount (modular) IDU-ODU connectivity, 1+0 configuration

### HW_CFG_01, Ethernet cable connectivity, ODU is a separate chassis

This configuration follows a split-type architecture, where the outdoor unit (MW ODU) is physically separated from the indoor unit (IDU). The IDU is of modular design and may include one or more containers hosting pluggable functional boards. In the considered configuration, the interconnection between the IDU and the ODU is provided via an Ethernet interface. For inventory modelling purposes, the MW ODU is represented as a separate chassis  with its own inventory scope.

Figure 3 HW_CFG_01

### HW_CFG_02, IF cable connectivity, ODU is a module and child component of the indoor

This configuration follows a split-type architecture with IF cable connectivity between the indoor unit (IDU) and the microwave outdoor unit (MW ODU). The IDU is again of modular design and may comprise one or more containers hosting pluggable functional boards. In the considered configuration, the interconnection between the IDU and the MW ODU is provided via an IF cable interface. For inventory modelling purposes, the MW ODU is represented as a module and modelled as a child component of the IDU, within the same chassis and inventory scope.

Figure 4 HW_CFG_02

## Compact (non-modular) IDU-ODU connectivity, 1+0 configuration

### HW_CFG_03, Ethernet cable connectivity, ODU is a separate chassis

This configuration follows a split-type architecture but this time with a fixed (compact, non-modular) indoor unit (IDU), where the IDU is represented as a chassis with a fixed set of physical ports. The RJ45 Ethernet ports are modelled as fixed ports of the chassis, as they are permanently integrated and not field-replaceable. SFP cages are modelled as containers capable of hosting pluggable transceiver components. IF/modem interfaces are modelled as containers representing pluggable/activatable modem functions within the IDU. Unlike the modular IDU representation, this configuration does not include container slots, since no removable board-level containers are present. In this case, the interconnection between the IDU and the MW ODU is provided via an Ethernet interface. For inventory modelling purposes, the MW ODU is represented as a separate chassis with its own inventory scope.

Figure 5 HW_CFG_03

### HW_CFG_04, IF cable connectivity, ODU is a module and child component of the indoor

This configuration follows a split-type architecture with a fixed (compact, non-modular) indoor unit (IDU), in which the interconnection between the IDU and the microwave outdoor unit (MW ODU) is provided via an IF cable interface. For inventory modelling purposes, the MW ODU is represented as a module and modelled as a child component of the IDU, within the same chassis and inventory scope.

Figure 6 HW_CFG_04

# Test Configurations

## Introduction

Each network equipment vendor provides a pair of network elements, consisting of a transmitter (local node) and a receiver (remote node), representing both ends of a millimetre wave or microwave link.

The physical connection of the RF units of the radio between local and remote node, as well as between the vendors, is realized with coaxial cable or waveguide plus attenuators; no antenna and no free space radiation is allowed.

The links are arranged in a linear topology to enable an end-to-end service to be configured.

Each network element is connected to the adjacent one via an optical fibre connection through a patch panel. The first and the last network elements in the total chain will be connected to a packet traffic generator/analyzer, also via optical fibre connections.

The Single Mode test configuration shall be used for conformance testing. The other test configurations shall be uused for interoperability testing.

## Single Mode

This mode consists of a single equipment vendor and the Tester and the Traffic Generator.

Figure 7: CB Single Mode

Figure 8: PB Single Mode

Figure 9: PEB Single Mode

## Dual Mode

This mode consists of two equipment vendors and the Tester and the Traffic Generator.

Figure 10: E2E Dual Mode

Figure 11: Cascade Dual Mode

## Chain Mode

The following figure defines the Chain mode with three participating vendors. More vendors may be integrated to the chain, by adding them one by one to the end of the chain.

The figure below shows the E2E Chain Mode, where only the NEs on the chain end were configured as PEB. The rest of the NEs were configured as PB. This configuration is typically deployed in networks where S-VLANs are transported from one end to the other end of the chain.

Figure 12: E2E Chain Mode

The figure below shows the Cascade Chain Mode, where all NEs were configured as PEB. This configuration is mostly used when it is necessary to manage several C-VLANs in every NE.

Figure 13: Cascade Chain Mode

# Use Cases and Tests

## Use Case 1: Network and Service Auto-discovery

### Functionality description

Network and Services auto-discovery is a function that allows to automatically detect and connect new microwave nodes to a management system and to retrieve inventory information from the node.

### Inventory or Network and services auto-discovery tests

Test Configuration:

Single Mode.

Prerequisite:

Setup a microwave link according to one of the HW profiles defined in clause 5.

The tests may be executed with tagged-traffic or untagged-trafic.

Table 1 Use Case 01, Single Mode, Test Overview

| Test ID | Status | Test Objective |
| --- | --- | --- |
| INV_HW_01 | M | HW Inventory |
| INV_SW_01 | M | SW Inventory |
| INV_CAR_01 | M | Microwave Carrier Inventory |
| INV_ETH_CAR_01 | M | Ethernet Carrier Inventory |
| INV_SRV_01 | M | Customer Bridge Mode Inventory |
| INV_NET_01 | M | LLDP Neighbor Inventory |

## USE CASE 2: Service Provisioning

### Service Description

E2E L2 Service Provisioning (creation, modification, and deletion):

An E-LINE service is configured E2E, traversing a chain of different Vendors' equipment.

Provider Bridge and Provider Edge mode shall be tested.

### End to End L2 service provisioning tests in Single Mode

Test Configuration:

Single Mode.

Prerequisite:

Setup a microwave link according to one of the HW profiles defined in clause 5.

The tests may be executed with tagged-traffic or untagged-trafic.

Table 2 Use Case 02, Single Mode, Test Overview

| Test ID | Status | Test Objective |
| --- | --- | --- |
| PRO_CUS_CRE_01 | M | C-VLAN Customer Bridge Mode - Creation |
| PRO_CUS_DEL_01 | M | C-VLAN Customer Bridge Mode - Deletion |
| PRO_BRD_CRE_01 | M | S-VLAN Provider Bridge Mode - Creation |
| PRO_BRD_DEL_01 | M | S-VLAN Provider Bridge Mode – Deletion |
| PRO_EDG_CRE_01 | M | S-VLAN Provider Edge Bridge Mode - Creation |
| PRO_EDG_DEL_01 | M | S-VLAN Provider Edge Bridge Mode - Deletion |

Table 3 Use Case 02, Single Mode,Test Descriptions

| Test ID | Status | Test Step | Description |
| --- | --- | --- | --- |
| PRO_CUS_CRE_01 | M | Step 1 | Verify no configurable services |
|  |  | Step 2 | Create a C-VLAN service in Customer bridge mode |
|  |  | Step 3 | Verify that service is created & traffic is normal (TG) Verify Service Repository |
| PRO_CUS_DEL_01 | M | Step 4 | Delete service |
|  |  | Step 5 | Verify service is deleted & traffic is cut (TG) |
| PRO_BRD_CRE_01 | M | Step 1 | Verify no configurable services |
|  |  | Step 2 | Create an S-VLAN based service carrying 2 C-VLANs in Provider Bridge mode |
|  |  | Step 3 | Verify that service is created & traffic is normal |
| PRO_BRD_DEL_01 | M | Step 4 | Delete service |
|  |  | Step 5 | Verify service is deleted & traffic is cut |
| PRO_EDG_CRE_01 | M | Step 1 | Verify no configurable services |
|  |  | Step 2 | Create an S-VLAN based service carrying 2 C-VLANs in Provider Edge Bridge mode |
|  |  | Step 3 | Verify that service is created & traffic is normal |
| PRO_EDG_DEL_01 | M | Step 4 | Delete service |
|  |  | Step 5 | Verify service is deleted & traffic is cut |

### End to End L2 service provisioning tests in Dual Mode

Test Configuration:

Dual Mode.

Prerequisite:

Setup a microwave link according to one of the HW profiles defined in clause 5.

The tests may be executed with tagged-traffic or untagged-trafic.

Table 4 Use Case 02, Dual Mode, Test Overview

| Test ID | Status | Test Objective |
| --- | --- | --- |
| E2E |  |  |
| INV_NET_01 | M | LLDP Neighbor Inventory |
| PRO_DM_E2E_CRE_01 | M | S-VLAN – E2E - Creation |
| PRO_DM_E2E_DEL_01 | M | S-VLAN - E2E – Deletion |
| Cascade |  |  |
| INV_NET_01 | M | LLDP Neighbor Inventory |
| PRO_DM_CAS_CRE_01 | M | S-VLAN – Cascade - Creation |
| PRO_DM_CAS_DEL_01 | M | S-VLAN - Cascade – Deletion |

Table 5 Use Case 02, Dual Mode, Test Descriptions

| Test ID | Status | Test Step | Description |
| --- | --- | --- | --- |
| E2E |  |  |  |
| INV_NET_01 | M | Step 1 | Neighbor Inventory |
| PRO_DM_E2E_CRE_01 | M | Step 2 | Vendor 1: PEB + PB |
|  |  | Step 3 | Vendor 2: PB + PEB |
|  |  | Step 4 | Verify that service is created & traffic is normal (TG) Verify Service Repository |
| PRO_DM_E2E_DEL_01 | M | Step 5 | Delete service |
|  |  | Step 6 | Verify service is deleted & traffic is cut |
| Cascade |  |  |  |
| INV_NET_01 | M | Step 1 | Neighbor Inventory |
| PRO_DM_CAS_CRE_01 | M | Step 2 | Vendor 1: PEB + PEB |
|  |  | Step 3 | Vendor 2: PEB + PEB |
|  |  | Step 4 | Verify that service is created & traffic is normal (TG) Verify Service Repository |
| PRO_DM_CAS_DEL_01 | M | Step 5 | Delete service |
|  |  | Step 6 | Verify service is deleted & traffic is cut |

### End to End L2 service provisioning tests in Chain Mode

The present clause describes exemplarily the test configuration for three vendors. This test configuration may be extended with more vendors.

Prerequisite:

Setup a microwave link according to one of the HW profiles defined in clause 5.

The tests may be executed with tagged-traffic or untagged-trafic.

Table 6 Use Case 02, Chain Mode, Test Overview

| Test ID | Status | Test Objective |
| --- | --- | --- |
| E2E |  |  |
| INV_NET_01 | M | LLDP Neighbor Inventory |
| PRO_CH3_E2E_CRE_01 | M | S-VLAN – E2E - Creation |
| PRO_CH3_E2E_DEL_01 | M | S-VLAN - E2E – Deletion |
| Cascade |  |  |
| INV_NET_01 | M | LLDP Neighbor Inventory |
| PRO_CH3_CAS_CRE_01 | M | S-VLAN – Cascade - Creation |
| PRO_CH3_CAS_DEL_01 | M | S-VLAN - Cascade – Deletion |

Table 7 Use Case 02, Chain Mode, Test Descriptions

| Test ID | Status | Test Step | Description |
| --- | --- | --- | --- |
| E2E |  |  |  |
| INV_NET_01 | M | Step 1 | Neighbor Inventory |
| PRO_CH3_E2E_CRE_01 | M | Step 2 | Vendor 1: PEB + PB |
|  |  | Step 3 | Vendor 2: PB + PB |
|  |  | Step 4 | Vendor 3: PB + PEB |
|  |  | Step 5 | Verify that service is created & traffic is normal (TG) Verify Service Repository |
| PRO_CH3_E2E_DEL_01 | M | Step 6 | Delete service |
|  |  | Step 7 | Verify service is deleted & traffic is cut |
| Cascade |  |  |  |
| INV_NET_01 | M | Step 1 | Neighbor Inventory |
| PRO_CH3_CAS_CRE_01 | M | Step 2 | Vendor 1: PEB + PEB |
|  |  | Step 3 | Vendor 2: PEB + PEB |
|  |  | Step 4 | Vendor 3: PEB + PEB |
|  |  | Step 5 | Verify that service is created & traffic is normal (TG) Verify Service Repository |
| PRO_CH3_CAS_DEL_01 | M | Step 6 | Delete service |
|  |  | Step 7 | Verify service is deleted & traffic is cut |

## USE Case 3: Performance Analysis & Prediction

### Functionality description

Radio performance degradation, unavailability and/or outage can be due to:

Rain Attenuation.

Diffraction Loss (obstructed LOS).

Atmospheric Multipath Fading:

Refraction: flat fading, selective fading.

Ducting.

Terrain Reflections (multipath).

RF Interference.

IF Interference.

In order to simulate the above phenomena, this use case queries specific performance metrics such as RSL (Received Signal Level), Tx (Transmit Power), SNR (Signal to Noise Ratio), operates with minimum power and checks for Threshold Alarm notifications.

### Performance Analysis & Prediction tests

Test Configuration:

Single mode.

Prerequisite:

Setup a microwave link according to one of the HW profiles defined in clause 5.

No active alarms related to this specific radio configuration.

The tests may be executed with tagged-traffic or untagged-trafic.

Table 8 Use Case 03, Single Mode, Test Overview

| Test ID | Status | Test Objective |
| --- | --- | --- |
| PER_CAR_01 | M | Query performance metrics |
| PER_CAR_02 | M | Operation with minimum Tx power |
| PER_CAR_03 | O | Threshold Alarm Notification |
| PER_CAR_04 | O | Re-establish degraded link |

Table 9 Use Case 03, Single Mode, Test Descriptions

| Test ID | Status | Test Step | Description |
| --- | --- | --- | --- |
| PER_CAR_01 | M | Step 1 | Query specific performance metrics: RSL, Tx Power, SNR/MSE |
|  |  | Step 2 | Verify successful communication of the metrics to the Test System |
| PER_CAR_02 | M | Prerequisite | Operate again with max Transmit power |
|  |  | Step 1 | Reduce Tx power to minimum value (local node) |
|  |  | Step 2 | Query specific performance metrics: RSL, Tx Power, SNR/MSE |
|  |  | Step 3 | Verify degraded performance (RSL degraded) on remote node Response received without errors |
| PER_CAR_03 | O | Prerequisite | Operate again with max Transmit power |
|  |  | Step 1 | Set Threshold for RSL value |
|  |  | Step 2 | Reduce Tx power to minimum value (local node) |
|  |  | Step 3 | When the threshold is surpassed then corresponding Threshold-Alarm will be generated. The threshold-alarm shall be shown in the alarm-list of the network element. |
|  |  | Step 4 | The threshold alarm is communicated to the Test System by means of NETCONF notifications |
|  |  | Step 5 | Query RSL value on remote node Verify degraded performance |
| PER_CAR_04 | O | Prerequisite | Perform above test first (PER_CAR_03) |
|  |  | Step 1 | The Test System sends a command to increase the Tx power to maximum |
|  |  | Step 2 | Verify correct link operation |

## Use Case 4: Smart Alarm Analysis & Fault Prediction

### Functionality description

This use case tests only the API which enables the Use Case 4 features. It can be considered as a prerequisite for implementing at a later stage the Use Case "Smart Alarm Analysis & Fault Prediction" defined in [1].

"Carrier down alarm" is created by muting the ODU. The alarm list is queried, or optionally, a network node sends a notification. The controller reacts upon this notification and takes the appropriate steps to recover the link.

### Smart Alarm Analysis & Fault Prediction tests

Test Configuration:

Single mode.

Prerequisite:

Setup a microwave link according to one of the HW profiles defined in clause 5.

No active alarms related to this specific radio configuration.

The tests may be executed with tagged-traffic or untagged-trafic.

Table 10 Use Case 04, Single Mode, Test Overview

| Test ID | Status | Test Objective |
| --- | --- | --- |
| ALA_CAR_01 | M | Query alarm list |
| ALA_CAR_02 | M | New alarm in alarm list due to broken link |
| ALA_CAR_03 | O | Alarm Notification due to broken link |
| ALA_CAR_04 | O | Re-establish broken link |

Table 11 Use Case 4, Single Mode, Test Descriptions

| Test ID | Status | Test Step | Description |
| --- | --- | --- | --- |
| ALA_CAR_01 | Mandatory | Step 1 | Query alarms (Get query command - alarm inventory & alarm list) |
|  |  | Step 2 | Verify Communication of Alarms (no link related alarms in the alarm list) |
| ALA_CAR_02 | Mandatory | Step 1 | Query the alarms (alarm list) |
|  |  | Step 2 | Mute one ODU to break the link - local NE |
|  |  | Step 3 | Query again the alarms (alarm list) & verify degraded performance (on remote NE) |
| ALA_CAR_03 | Optional | Prerequisite | Recover operation on the link |
|  |  | Step 1 | Mute one ODU to break the link |
|  |  | Step 2 | Verify degraded performance via Netconf notifications received by the Test System |
| ALA_CAR_04 | Optional | Prerequisite | Recover operation on the link |
|  |  | Step 1 | Mute one ODU (local NE) to break the link |
|  |  | Step 2 | Verify degraded performance (by the radio link down alarm) - (remote NE) |
|  |  | Step 3 | The Test System sends a command/notification to unmute the ODU |
|  |  | Step 4 | Verify correct link operation |

# mWT SDN SBI Hardware Inventory Profile

Microwave wireless backhaul networks constitute a critical part of modern transport infrastructures. Their integration into an open SDN architectures has historically been constrained by the lack of standardised and interoperable hardware inventory models. As a consequence, SDN Controllers have been required to interpret vendor-specific and inconsistent data structures, leading to an increased integration effort, reduced automation potential, and limited portability of SDN applications.

This limitation has been addressed by defining a hardware inventory profiles for the SDN Southbound Interface, expressed through YANG models aligned with the relevant IETF specifications. These profiles are tailored to real microwave configurations and operational deployment scenarios. The objective is to ensure that the microwave domain may be represented in a uniform, vendor-neutral, and interoperable manner within open, programmable, multi-vendor SDN transport networks.

The hardware inventory profiles are defined in “SDN SBI Wireless Transport – Profiles for HW Inventory Use Cases” [3]. This work item specifies the principles, objectives, and technical framework, which addresses the definition of standardized hardware inventory models for microwave wireless backhaul at the SDN Southbound Interface.

For the 5th mWT SDN Plugtests event, use-case-specific parameter sets were developed based on the work item “SDN SBI Wireless Transport – Profiles for HW Inventory Use Cases” [3]. The attached document “mWT SDN SBI Hardware Inventory File” contains the use-case-specific parameter sets.